## 6. Defect Detection & Quantification

Defect detection was implemented using a Six Sigma-based statistical approach, where observations were flagged as anomalous if any feature exceeded ±3 standard deviations (±3σ) from the mean.

The analysis was conducted at two levels:

### (A) Sensor-Level Detection

Each observation was marked defective if any sensor feature violated the ±3σ rule.

From the results:

- Total defective observations: 1279

- Total normal observations: 286

- Sensor-level defect rate = 0.8162 (81.62%)

Interpretation:

- The system is highly sensitive, detecting most abnormal conditions

- However, this leads to over-detection (many false positives)

### (B) PCA-Level Detection (Validation Layer)

PCA-transformed features were used to detect anomalies in reduced-dimensional space.

- PCA defect rate = 0.3848 (38.48%)

Interpretation:

- PCA reduces noise and captures structural anomalies

- Produces fewer but more meaningful defects

### Validation Using Ground Truth Labels

#### Sensor Model Confusion Matrix:

- TP = 86

- FP = 1193

- FN = 18

- TN = 270

- Recall = 82.7%

- Precision = 6.7%

Interpretation:

- Very high recall → captures most defects

- Extremely low precision → many false alarms

#### PCA Model Confusion Matrix:

- TP = 54

- FP = 549

- FN = 50

- TN = 914

- Recall = 51.9%

- Precision = 8.9%

Interpretation:

- Lower recall (misses defects)

- Better precision (fewer false alarms)

### Key Insight

| Model | Strength | Weakness |
| --- | --- | --- |
| Sensor (3σ) | High recall (detects most defects) | Too many false positives |
| PCA | Reduced false alarms | Misses some defects |

This validates your statement:
PCA helps separate real defects from noise

## 7. Process Capability Analysis

Capability analysis was performed on selected critical sensors identified using:

- High ±3σ violation frequency

- Large difference between defective vs normal products

Selected sensors included:

feature_103, feature_510, feature_348, feature_577, feature_59, feature_431, feature_38, feature_574, feature_576, feature_573

### Results

- Cp = 1.0

- Cpk = 1.0 (for all selected sensors)

### Critical Interpretation

These values are NOT meaningful

- Data was standardized (mean = 0, std = 1) before analysis

- Specification limits were artificially set to ±3σ

Therefore:

- Cp and Cpk = 1 by construction

- They do NOT reflect real manufacturing capability

### Conclusion

- Capability analysis on standardized data is not valid for real-world conclusions

- However, it demonstrates correct methodology

## 8. Statistical Process Control (SPC)

SPC was applied using rolling statistics to simulate real-time monitoring.

### X-bar Chart (Mean Monitoring)

From our plot :

Observations:

- Multiple points exceed control limits (±3σ)

- Significant spikes:

  - Around index ~700 (+10)

  - Around ~850 (−7)

  - Around ~1150

Interpretation:

- Presence of out-of-control conditions

- Indicates abnormal process behaviour

### Process Drift

- Rolling mean shows:

  - Gradual downward trend

  - Later stabilization

Interpretation:

- Process drift → possible calibration issue or wear

### R Chart (Variability Monitoring)

From our plot:

Observations:

- Normal variability: ~0.5–1.0

- Sudden spikes:

  - Around 700 (~2.3)

  - Around 850 (~1.7)

  - Around 1150 (~1.6)

Interpretation:

- Sudden increases in variability

- Indicates process instability

### Overall SPC Insight

- Process is not fully stable

- Contains:

  - Out-of-control points

  - Drift

  - Variability spikes

## 9. Six Sigma Framework (DMAIC)

### Define

- Objective: Detect anomalies and improve process quality

- Focus: Reduce defect rate and instability

### Measure

- Sensor defect rate: 81.62%

- PCA defect rate: 38.48%

- Recall (sensor): 82.7%

- Precision (sensor): 6.7%

### Analyse

- High false positives → noise in high-dimensional space

- PCA improves signal clarity

- Key unstable sensors identified

### Improve

- Use PCA filtering before defect detection

- Focus on critical sensors (top violation + impact sensors)

- Reduce noise in measurement system

### Control

- Implement SPC monitoring:

  - X-bar chart → detect shifts

  - R chart → detect variability

- Deploy real-time anomaly alerts

## 10. Final Operational Impact

The system enables a transition from:

- Reactive maintenance towards a Proactive quality control

Key benefits:

- Early detection of process issues

- Reduced defect rates

- Improved yield and consistency

- Efficient resource allocation

# 11. Actionable Insights

### 1. Focus on Critical Sensors

Key problematic sensors:

feature_103, feature_510, feature_348, feature_577, feature_59

Action:

- Monitor these in real-time

- Prioritize maintenance

### 2. Address Process Instability

SPC shows:

- Sudden spikes

- Drift

Action:

- Investigate machine calibration

- Check environmental factors

### 3. Implement Real-Time SPC Dashboard

- X-bar chart → detect shifts

- R chart → detect variability

Enables:

- Immediate intervention

### 4. Improve Data Quality

- High noise in raw sensors

Action:

- Sensor recalibration

- Noise filtering

## 12. Expected Outcomes

- Automated anomaly detection system

- Reduced false alarms via PCA validation

- Better understanding of process instability

- Improved decision-making through statistical monitoring

- Integration of ML with Six Sigma